CONTRACTOR

SCOPE OF WORK

xxxx Xxxxxxxx Drive

Marietta, GA  30008

Floor Plan Reconfiguration

- Configuration as illustrated below

- Add power outlets as necessary to accommodate for those removed

- Add light switches as necessary to accommodate for those removed

Pre-Rehab:

Post-Rehab:

SYSTEMS

Plumbing

- Replace Hot Water Heater to code

- Water pressure is way too high (95 PSI).  Investigate/remediate

Electrical

- Repair wire splices in attic (add electrical boxes or rewire)

- Put cover on breaker box

- Mis-wired circuits in Dining Room/Living Room (not all outlets are hot; some have no ground)

HVAC

- Add supply/return ducts in the bonus room (converted garage)

- Add supply/return ducts in the laundry room

- Repair/replace corroded furnace venting/duct work in attic and crawlspace

- Repair loose insulation in crawl space

- Repair points of air leakage in crawl space

- Add air supply registers and return grill in Laundry Room (depending on cost)

Other

- Repair door-bell

INTERIOR

General

- Patch all walls and trim in preparation for paint

- Replace all interior entry doors with new 6-panel doors and new door hardware

- Replace all interior closet doors with new slab doors and new door hardware

- Replace all vent covers where missing

- Fix/replace outlets/covers as necessary (ensure matching colors and styles)

- Fix/replace switches/switch-plate covers as necessary (ensure matching colors and styles)

Painting

- Paint entire interior with three color paint scheme (walls, trim, ceiling)

Kitchen

- Keep sink/faucet/disposal

- Replace all cabinets

- Replace countertops

- Tile backsplash behind counters

- Replace all appliances (stove, microwave, fridge, dishwasher)

- Add L-Shaped island between kitchen and living room (w/cabinets and countertops)

- Replace light fixture with 4 cans recessed lighting

- Replace aluminum window with new vinyl window

- Replace vinyl flooring with ceramic tile

Dining Room

- Remove sliding glass door and replace with French Doors leading onto back deck

- Remove faux beams on ceiling

- Replace vinyl flooring with laminate hardwood

- Remove paneling and sheetrock room

- Replace light/fan combination with chandelier

Entryway

- Replace front entry door

- Replace vinyl flooring with laminate hardwood

- Repair light switches in entryway

- Replace light fixture with 1 can of recessed lighting

Living Room

- Lay laminate hardware floors

- Add 6 cans of recessed lighting

Converted Garage/Bonus Room

- Level floor (add new subflooring?)

- Add additional HVAC supply/return vents for better temperature regulation

- Remove drop ceiling

- Remove paneling

- Sheetrock entire room

- Lay carpet

- Frame and sheetrock entryway leading to kitchen/laundry

- Replace archway entry from living room with French Doors

Laundry Room

- Fix/replace subfloor

- Sheetrock entire room

- Add HVAC supply/return vents for better temperature regulation

- Properly case and replace door to outside, including sill

- Lay vinyl flooring

- Replace light fixture

- Install washer/dryer

Hallway

- Lay carpet

- Replace light fixture

- Cover doorbell transformer

Hallway Bathroom

- Replace floor with ceramic tile (including small closet)

- Add shelving in small closet

- Replace toilet

- Replace vanity/sink

- Replace mirror

- Replace light fixture

- Clean tub

- Add shower curtain bar

Master Bathroom

- Keep toilet

- Replace aluminum window with new vinyl window

- Remediate mold in shower (may need to replace sheetrock)

- Caulk around crown molding to hide cracks (or replace crown molding)

- Replace floor with ceramic tile

- Replace fiberglass shower base

- Use same ceramic tile from floor on shower walls

- Replace vanity/sink

- Replace mirror

- Replace light fixture

- Replace shower frame and door

- Replace shower water knobs/valves

Bedroom #1

- Lay carpet

- Replace light/fan fixture

- Replace hardware on double closet doors

Bedroom #2

- Lay carpet

- Replace aluminum window with new vinyl window

- Replace light/fan fixture

Master Bedroom

- Lay carpet

- Replace light/fan fixture

- Replace both aluminum windows with new vinyl windows

- Replace hardware on double closet doors

EXTERIOR

Paint

- Paint entire non-brick exterior (including trim/shutters)

- Paint front porch concrete

- Pressure-wash brick to try to remove existing paint

Roof

- Repair damaged gutters

- Fix damaged/missing downspout sections

- Clean gutters

- Repair soffits/fascia board/trim around entire exterior as necessary

Front Steps/Deck

- Pressure wash concrete

Back Deck

- Remove existing concrete deck for better drainage away from house

- Build deck off French Doors leading from Dining Room

- Deck should be the size of the concrete patio

- Steps leading off deck to back-yard

Landscaping

- Grade land/dirt away from house where necessary to ensure proper grading

- Demo shed in back yard

- Remove existing foundation (or whatever is there) next to shed to allow grass to grow

- Trim all bushes

- Trim lower branches off front trees (to display the house)

- Clean-up the backyard

Other

- Fix flashing between brick/wood under front windows (replace with sealant/caulking)